На разположение на: BoL, TzP, RoM + R&D, ATInductive

Техническа спецификация

Колаборативен робот за боядисване на кожени детайли и/или заваряне.

История:

Проектът е започнал като комбинация от три предходни проекта:
• Проект 25067 - Система за боядисване на кожени детайли  
• Проект 25081 - Автоматизирана система за заваряване
• Проект 24701 - Колаборативна роботика за индустриални приложения
Текущата версия обединява функционалността на тези системи в единна платформа с модулен софтуер, базиран на ROS 2 и Python.

| Дата | Име | Описание |
| --- | --- | --- |
| 29.07.2025 | AtD | Съставяне на документа |
| 14.01.2026 | AtD | Допълване на документация и въвеждане на нови функционалности, изисквания и други. |
| 21.01.2026 | AtD | Въвеждане на архитектура, текущи и бъдещи функционалности, основна структура на софтуерът. |

Съдържание:

1	Изисквания	3

2	Проверка на изискванията	...

3	Идеи за реализация	...

4	Реализация	...

5	Тестове и проверка на функционалността	...

6	Отворени точки	...

Фигури:

Error! No table of figures entries found.

Таблици:

Error! No table of figures entries found.

Използвани съкращения:

Изисквания

Разработване на автаматизиран колаборативен робот който ще има възможността да изпълнява множество функции чрез унифициран софтуер на основата на проекти 25067, 25081 и 24701.

  - Общ вид за добиване на визуална представа на идеята за машината:

  - :

  - :

  - Софтуерно осигуряване:

• Операционна система: Ubuntu 22.04 LTS
• ROS 2 (Robot Operating System) Humble Hawksbill  
• MoveIt 2 за планиране на траектории
• Python 3.10+ за основна логика на приложението
• PyQt6 за графичен интерфейс
• OpenCV за обработка на изображения и компютърна визия
• NumPy, SciPy за математически изчисления
• python-docx за документация

  - :

  - :

  - :

1.7.1 ) :

Проверка на изискванията

2.1) Конструкция:

• Проверка на стабилност на монтажната рама и конструктивен профил
• Валидиране на захващания на робота към рамата
• Тестване на достъпността до работната зона
• Проверка на кабелното управление и защита
• Верификация на защитни елементи и шумоизолация

2.2) Захранване:

• Проверка на електрическата мрежа (220V AC, 50Hz)
• Валидиране на захранващи кабели и свързвания
• Тестване на предпазители и електрически защити
• Проверка на заземяване и електрическа безопасност
• Верификация на стабилност на захранването при пълно натоварване

2.3) Конструктивен дизайн

2.4) Оперативни изисквания:

• Работна температура: 15-30°C (оптимално: 20-25°C)
• Влажност: 20-80% относителна (без кондензация)
• Прецизност на позициониране: ±0.1mm (fine search режим)
• Скорост на изпълнение: Конфигурируема (5-100% от максимална)
• Време за калибрация: < 5 минути (автоматична 5-точкова)
• Цикъл на работа: Непрекъснато (с редовна проверка)
• Осветление: Стабилно, без директни отражения

2.5) Софтуерно осигуряване:

• Проверка на ROS 2 комуникация между компоненти (Topics, Services, Actions)
• Валидиране на MoveIt 2 планиране на траектории и collision detection
• Тестване на камерна калибрация и трансформации камера-робот
• Верификация на модулите за боядисване (painting_operation.py)
• Проверка на GUI отзивчивост и стабилност на PyQt6 интерфейса
• Тестване на визионна система (контури, ArUco маркери)
• Проверка на Modbus комуникация с периферни устройства
• Валидиране на state management и MessageBroker

- Тестване на функционалността на управляващото устройство, включително операционна система, и камера.

- Тестване на калибрацията.

2.7) Функционалност и характеристики:

• Автоматично разпознаване на контури чрез компютърна визия (OpenCV)
• Адаптивно боядисване с контрол на налягане (force-based compensation)
• Прецизно проследяване на ръбове с динамична ротация
• Поддръжка на прости форми (правоъгълници, квадрати)
• Поддръжка на сложни форми (кръгове, овали, фаски, закръгления)
• Калибрация камера-робот чрез ArUco маркери (автоматична)
• Модулна архитектура за лесно добавяне на нови функции
• Двустепенно търсене на контакт с висока прецизност (coarse + fine)
• Усредняване на сила с премахване на изключения (outlier rejection)
• Проверка на консистентност и retry логика
• Поддръжка на XY/rZ и YZ/rY работни режими
• Защита срещу осукване на кабели(ограничение до +/-360° на J6)

2.8) Управление. Хардуерни компоненти

- Хардуерните компоненти да отговарят на изисканото и да са подходящи и ценовосъобразени с търсения краен резултат.

2.9) Поддръжка:

• Редовна проверка на калибрацията (месечно или при промяна на работна позиция/височина)
• Почистване на камерата и работна зона (седмично)
• Проверка на налягане, вакуум помпа и консумативи (седмично)
• Софтуерни актуализации и security patches (при необходимост)
• Backup на настройки и калибрационни данни (седмично)
• Проверка на кабели и механични елементи (месечно)
• Log файлове анализ за грешки (при проблеми)

2.10) Документация:

• Техническа спецификация (този документ)
• Потребителско ръководство с инструкции за работа
• API документация на софтуерни модули (в код)
• Електрически схеми и кабелни диаграми
• Процедури за калибрация (стъпка по стъпка)
• Списък с резервни части и консумативи
• Troubleshooting guide (основни проблеми и решения)

- Проверка за подходящо етикетиране и документация, включително ръководство с инструкции.

Идеи за реализация

Хардуерна схема за реализация.

Да се изработи изцяло разглобяема конструкция от конструктивен профил.

Вакуум помпата и електрическите компоненти да бъдат вградени в корпуса като бъдат разделени едни от други.

Вакуум помпата да бъде шумоизолирана.

Да има лесен достъп и работа с машината като се предвидят поне два вида на управление.

Никъде по машината да няма свободен достъп на електрически елементи или кабели.

Софтуерна схема за реализация.

АРХИТЕКТУРА НА СОФТУЕРА
1. Основни слоеве (Layered Architecture):
   • Core Layer - Базова функционалност и управление на състояния
   • Communication Layer - API Gateway и ROS 2 интеграция  
   • Application Layer - Специфични приложения (боядисване, заваряване)
   • Frontend Layer - Графичен интерфейс (PyQt6)
   • Plugins Layer - Модулни допълнения
2. Ключови модули и файлове:
   2.1) Vision System (modules/VisionSystem/):
      • calibration/ - Камерна калибрация
      • handlers/contour_detection_handler.py - Детекция на контури
      • handlers/aruco_detection_handler.py - ArUco маркери
      • Трансформация координати камера-робот (perspective transform)
   2.2) Painting Operation (applications/edge_painting_application/):
      • painting_operation.py (3536 lines) - Основна логика за боядисване
        - _find_contact_point_precise() - Двустепенно търсене на контакт
        - _get_averaged_force() - Усредняване на сила с outlier rejection
        - _check_force_consistency() - Проверка на стабилност
        - _convert_contour_to_robot_path() - Преобразуване контур → траектория
        - _execute_contour_following_painting() - Проследяване на криви
        - _execute_corner_painting_xy_rz() - Правоъгълно боядисване (XY режим)
        - _execute_corner_painting_yz_ry() - Вертикално боядисване (YZ режим)
      • glue_operation.py - Glue dispensing функционалност
      • application.py - Application wrapper
   2.3) Robot Control (core/):
      • controllers/robot/base_robot_controller.py - Абстрактен контролер
      • model/robot/fairino_robot.py - ZeroErr робот имплементация
      • services/robot_service/impl/base_robot_service.py - Robot service
      • MoveIt 2 интеграция чрез gui_bridge.py (ROS 2 workspace)
   2.4) State Management:
      • core/operation_state_management.py - IOperation интерфейс
      • core/application_state_management.py - Състояния (IDLE/RUNNING/PAUSED/ERROR)
      • modules/shared/MessageBroker.py - Pub/Sub за междумодулна комуникация
   2.5) Calibration System (modules/robot_calibration/):
      • newRobotCalibUsingExecutableStateMachine.py (559 lines)
      • Автоматична 5-точкова калибрация
      • states/axis_mapping.py - Axis mapping и валидиране
      • Homography изчисление и верификация
   2.6) Frontend (frontend/):
      • core/main_window/MainWindow.py (814 lines) - Главен прозорец
      • dialogs/OperationParamsDialog.py (736 lines) - Параметри
      • plugins/core/dashboard/ui/DashboardWidget.py - Оперативен екран
   2.7) Peripherals:
      • modules/modbusCommunication/ModbusController.py - Modbus I/O
3. Комуникационен протокол:
   • ROS 2 Topics - Реално време (camera feed, robot state)
   • ROS 2 Services - Синхронни заявки (move commands)
   • ROS 2 Actions - Дълги операции (trajectory execution)
   • Modbus RTU/TCP - Периферни устройства (релета, сензори)
   • Socket communication - Python GUI ↔ ROS 2 (gui_bridge.py)
4. Поток на данни при боядисване:
   Камера → Capture Frame → 
   Contour Detection (OpenCV) → 
   Camera→Robot Transform (Homography) → 
   Waypoint Generation → 
   Trajectory Planning (MoveIt 2) → 
   Path Execution (Robot Controller) → 
   Force Feedback → Position Adjustment
5. Конфигурационни параметри (OperationParamsDialog):
   Движение:
   • velocity: 5-100% (default: 20%)
   • acceleration: 5-100% (default: 20%)
   • z_offset: Височина над повърхността (default: 6.0mm)
   • lift_z: Височина при преместване (default: 200mm)
   • rotation_step: Стъпка при ротация в ъгли (default: 1.0°)
   Налягане:
   • target_force_x: Целева сила (default: 2.0N)
   • force_adjustment_gain: Gain за корекция (default: 0.1 mm/N)
   • force_deadband: Deadzone (default: 0.5N)
   Търсене на контакт:
   • max_search_distance: Макс. разстояние (default: 15mm)
   • search_step_size: Coarse стъпка (default: 0.5mm)
   • fine_search_step: Fine стъпка (default: 0.1mm)
   • contact_threshold: Праг за контакт (default: 1.0N)
   • contact_settle_time: Време за стабилизиране (default: 0.3s)
   Прецизност:
   • use_force_averaging: Усредняване на сила (default: True)
   • force_samples: Брой samples (default: 5)
   • use_gradual_approach: 2-степенно търсене (default: True)
   • force_consistency_check: Проверка стабилност (default: True)
   • max_contact_retries: Retry опити (default: 3)
   Contour Following:
   • enable_contour_following: Криви форми (default: True)
   • contour_simplification_epsilon: Douglas-Peucker (default: 2.0mm)
   • rotation_lookahead_points: Lookahead (default: 5)
6. Precision Enhancement система:
   Двустепенно търсене на контакт:
   Stage 1 (Coarse): 0.5mm стъпки до 40% от threshold
   Stage 2 (Fine): 0.1mm стъпки с 3 consecutive readings
   Force averaging:
   • 5 samples на 50ms интервал
   • Median Absolute Deviation (MAD) за outlier rejection
   • Keeps samples within 2*MAD от median
   Consistency validation:
   • std_dev < 0.3N за стабилен контакт
   • 3 consecutive stable readings required
   • Retry ако consistency check fails
   Exponential smoothing:
   • Alpha = 0.3 за smooth position changes
   • Max change = 0.5mm per step (rate limiting)
7. Сигурност и надеждност:
   • State machine с ясни transitions
   • Safety checks преди движение
   • Workspace boundary validation
   • Force limiting и monitoring
   • Emergency stop handling
   • Cable wrap prevention (360° cumulative tracking)
   • Error recovery и graceful degradation
8. Файлова структура:
src/
├── applications/                    # Application layer
│   └── edge_painting_application/
│       ├── painting_operation.py   # 3536 lines, main logic
│       ├── glue_operation.py
│       └── application.py
├── core/                           # Core functionality
│   ├── controllers/robot/
│   ├── services/robot_service/
│   ├── model/robot/
│   ├── operation_state_management.py
│   └── application_state_management.py
├── modules/                        # Reusable modules
│   ├── VisionSystem/
│   │   ├── calibration/
│   │   └── handlers/
│   ├── robot_calibration/
│   │   ├── states/
│   │   └── newRobotCalibUsingExecutableStateMachine.py
│   ├── modbusCommunication/
│   └── shared/
│       └── MessageBroker.py
├── frontend/                       # User interface
│   ├── core/main_window/
│   ├── dialogs/
│   └── plugins/
├── plugins/                        # Plugin system
│   └── core/
│       ├── dashboard/
│       └── settings/
└── libs/                          # External libraries
ROS 2 Workspace (ros2_ws/):
└── eRob_moveit/src/eRob_ROS2_MoveIt/pl25488/
    └── scripts/
        └── gui_bridge.py           # Socket bridge Python ↔ ROS 2
9. Изпълнение и threading:
   • Main GUI thread - PyQt6 event loop
   • Operation thread - Background за painting операции
   • ROS 2 threads - Spinner за callback processing
   • MessageBroker - Async pub/sub messaging
   • Socket communication - Non-blocking I/O

Реализация

РЕАЛИЗАЦИЯ НА СИСТЕМАТА
1. ХАРДУЕРНА КОНФИГУРАЦИЯ:
   1.1) Колаборативен робот:
      • Модел: ZeroErr 6-axis collaborative robot
      • Payload: До 5kg
      • Reach: ~900mm
      • Repeatability: ±0.05mm
      • Communication: Ethernet, Modbus TCP
   1.2) Vision система:
      • Индустриална камера (USB 3.0 или GigE)
      • Resolution: 1280x720 или по-висока
      • Frame rate: 30 fps minimum
      • Lens: Fixed focal length, подходящ FOV
      • Mounting: Статична позиция с clear view на работна зона
   1.3) Периферни устройства:
      • Вакуумна помпа за захващане на детайли
      • Pressure регулатор и сензори
      • Modbus I/O модул за релета (Y0 - vacuum pump)
      • Emergency stop бутон
      • Status индикатори (LED)
   1.4) Управляваща станция:
      • Industrial PC или workstation
      • OS: Ubuntu 22.04 LTS
      • RAM: 16GB minimum (32GB препоръчително)
      • CPU: 4+ cores, 2.5GHz+
      • Storage: 256GB SSD minimum
      • Network: Gigabit Ethernet
   1.5) Конструкция:
      • Алуминиев конструктивен профил
      • Разглобяема рама за лесен достъп
      • Шумоизолация за вакуумната помпа
      • Кабелно управление и защита
      • Заземяване и електрическа безопасност
      • Захранване 220V AC, 50Hz
2. СОФТУЕРНА ИМПЛЕМЕНТАЦИЯ:
   2.1) Core системи:
      • ROS 2 Humble Hawksbill - Middleware за robot comm.
      • MoveIt 2 - Motion planning framework
      • Python 3.10+ - Application logic
      • PyQt6 - GUI framework
      • OpenCV 4.x - Computer vision
      • NumPy/SciPy - Numerical computations
   2.2) Edge Painting Application:
      Основна функционалност:
      - Автоматично разпознаване на детайли
      - Контур detection и analysis
      - Camera-robot calibration
      - Траекторно планиране за боядисване
      - Force-based contact search
      - Adaptive pressure control
      - Поддръжка на прави и криви форми
      Precision features:
      - 2-stage contact detection (coarse + fine)
      - Force averaging (5 samples, MAD filtering)
      - Consistency checking (std_dev < 0.3N)
      - Retry logic (до 3 attempts)
      - Cumulative rotation tracking (360° limit)
      Painting modes:
      - Corner painting (XY/rZ) - Horizontal surfaces
      - Corner painting (YZ/rY) - Vertical surfaces
      - Contour following - Curved shapes
   2.3) Glue Dispensing (базова поддръжка):
      - Path following за glue dispensing
      - Flow control integration (планирано)
   2.4) Welding Application (планирано):
      - Trajectory generation за welding
      - Arc control integration
      - Multi-pass welding
3. ВИЗИОННА СИСТЕМА:
   3.1) Camera calibration:
      • Intrinsic calibration (checkerboard pattern)
      • Extrinsic calibration (camera-robot transform)
      • 5-point automatic calibration workflow
      • ArUco marker detection за reference
      • Homography calculation
      • Transform validation и error checking
   3.2) Contour detection:
      • Adaptive thresholding
      • Canny edge detection
      • findContours (OpenCV)
      • Contour filtering (area, aspect ratio)
      • approxPolyDP за simplification
      • minAreaRect за orientation
   3.3) Coordinate transformation:
      • cv2.perspectiveTransform()
      • Camera coordinates → Robot coordinates
      • TCP offset compensation
      • Z-height management
4. ПРЕЦИЗЕН КОНТРОЛ:
   4.1) Force estimation:
      • Simplified Jacobian transpose method
      • Joint efforts → Cartesian forces
      • Coefficients: J1*0.3 + J2*0.5 + J3*0.2
      • Real-time monitoring (50Hz typ.)
   4.2) Contact search:
      Stage 1 - Coarse (step=0.5mm):
      - Fast approach до 40% threshold
      - Averaged force (3 samples)
      - Consecutive increase detection
      Stage 2 - Fine (step=0.1mm):
      - Precise positioning
      - Full averaging (5 samples)
      - Consistency validation
      - 0.3s settling time
      - Automatic backoff към target force
   4.3) Pressure compensation:
      • Real-time X position adjustment
      • Gain: 0.1 mm/N (configurable)
      • Deadband: 0.5N
      • Max adjustment: 12mm
      • Optional rotation compensation
5. ИНТЕГРАЦИЯ:
   5.1) ROS 2 ↔ Python:
      gui_bridge.py (socket server):
      - Listens на port 5005
      - JSON command protocol
      - Async execution
      - Status callbacks
      Commands:
      - move_cartesian_path
      - get_current_position
      - get_joint_efforts
      - stop_motion
      - home
   5.2) Python ↔ GUI:
      MessageBroker pattern:
      - SystemTopics enum
      - Publish/Subscribe
      - Qt signals for UI updates
      - Thread-safe communication
6. ПОТРЕБИТЕЛСКИ ИНТЕРФЕЙС:
   6.1) MainWindow (main_window.py):
      • Central widget management
      • Menu bar, toolbar
      • Status bar с real-time info
      • Plugin loading и integration
   6.2) Dashboard (DashboardWidget.py):
      • Operation control (Start/Stop/Pause)
      • Camera feed visualization
      • Contour overlay
      • Status indicators
      • Quick params access
   6.3) Settings:
      • Robot configuration (HOME, PAINTING_POS)
      • TCP offsets (X, Y)
      • Camera calibration UI
      • Network settings
   6.4) Operation Parameters Dialog:
      • 2-column layout (1400x900)
      • Real-time param updates
      • Persist settings (JSON)
      • Reset to defaults
      • Keyboard shortcut (Ctrl+O)
7. НАСТРОЙКИ И КАЛИБРАЦИЯ:
   7.1) Robot settings:
      • HOME position teaching
      • PAINTING_POS teaching
      • TCP offset configuration
      • Joint limits
      • Velocity/acceleration limits
   7.2) Camera calibration workflow:
      1. Place calibration board
      2. Capture images (checkerboard)
      3. Compute intrinsic parameters
      4. Place ArUco markers
      5. Move robot to 5 calibration points
      6. Capture marker positions
      7. Compute homography
      8. Validate transform
      9. Save calibration data
   7.3) Operation calibration:
      • Contact force target adjustment
      • Search parameters tuning
      • Contour simplification epsilon
      • Rotation lookahead tuning
8. ДАННИ И PERSISTENCE:
   • Robot settings: JSON файлове
   • Calibration data: transformation matrices (NumPy)
   • Operation params: JSON (auto-save)
   • Logs: Text файлове с timestamps
   • Backup: Weekly automated backup script
9. PRODUCTION USAGE:
   Типичен работен цикъл:
   1. Power on система
   2. Launch ROS 2 nodes (gui_bridge)
   3. Start Python GUI application
   4. Load robot config
   5. Verify calibration (ако нужно - recalibrate)
   6. Place workpiece в camera view
   7. Adjust operation parameters
   8. Start painting operation
   9. Monitor execution
   10. Quality check резултат
   11. Repeat или modify params
10. PERFORMANCE METRICS:
   Измерени стойности:
   • Calibration time: 3-4 минути
   • Position accuracy: ±0.1mm (fine mode)
   • Force stability: ±0.3N (with averaging)
   • Contact success rate: >95% (with retry)
   • Cycle time: 30-120s (зависи от контур)
   • CPU usage: 40-60% (при активна операция)
   • Memory: ~800MB RAM
   • Frame processing: 25-30 fps

Тестове и проверка на функционалността

ТЕСТВАНЕ И ВАЛИДИРАНЕ
1. UNIT TESTING:
   1.1) Vision system:
      ✓ Camera calibration accuracy
      ✓ ArUco detection reliability
      ✓ Contour detection edge cases
      ✓ Coordinate transformation precision
   1.2) Force calculations:
      ✓ Force estimation от joint efforts
      ✓ Averaging algorithm (MAD filtering)
      ✓ Consistency checking
   1.3) Path generation:
      ✓ Waypoint calculation
      ✓ Rotation angle normalization
      ✓ Cumulative rotation tracking
2. INTEGRATION TESTING:
   2.1) ROS 2 ↔ Python:
      ✓ Socket communication stability
      ✓ Command serialization/deserialization
      ✓ Async callback handling
      ✓ Error propagation
   2.2) GUI ↔ Backend:
      ✓ MessageBroker pub/sub
      ✓ Parameter updates
      ✓ State transitions
      ✓ UI responsiveness
   2.3) Robot ↔ Vision:
      ✓ Calibration workflow
      ✓ Coordinate transform accuracy
      ✓ Real-time frame processing
   2.4) Modbus I/O:
      ✓ Relay control (vacuum pump)
      ✓ Input reading
      ✓ Connection recovery
3. FUNCTIONAL TESTING:
   3.1) Painting Operation - Rectangles:
      ✓ Detection на правоъгълни контури
      ✓ Corner identification (4 corners)
      ✓ Straightening alignment
      ✓ Edge tracing (top, right, bottom, left)
      ✓ 90° rotation при всеки ъгъл
      ✓ Force-based contact search
      ✓ Pressure compensation
      ✓ Return to home
      ✓ Vacuum pickup/dropoff
   3.2) Painting Operation - Complex shapes:
      ✓ Кръгове (полна 360° ротация - rejected)
      ✓ Овали (partial rotation)
      ✓ Закръглени ъгли (fillets)
      ✓ Фаски (chamfers)
      ✓ Неправилни polygons
      ✓ Contour simplification
      ✓ Rotation lookahead smoothing
      ✓ Cumulative rotation < 360° check
   3.3) Precision features:
      ✓ 2-stage contact search
        - Coarse: 0.5mm steps
        - Fine: 0.1mm steps
      ✓ Force averaging (5 samples)
      ✓ Outlier rejection (MAD)
      ✓ Consistency checking (std_dev < 0.3N)
      ✓ Retry logic (up to 3 attempts)
      ✓ Contact settle time (0.3s)
      ✓ Backoff към target force
   3.4) Calibration:
      ✓ 5-point camera-robot calibration
      ✓ ArUco marker detection
      ✓ Homography calculation
      ✓ Transform validation
      ✓ Axis mapping
      ✓ Persistence (save/load)
   3.5) GUI Functionality:
      ✓ Real-time camera feed (25-30 fps)
      ✓ Contour visualization overlay
      ✓ Parameter dialog (Ctrl+O)
      ✓ Start/Stop/Pause operations
      ✓ State indicators
      ✓ Error messages
      ✓ Settings persistence
4. PERFORMANCE TESTING:
   Измерени резултати:
   Време за операции:
   • Calibration: 3.5 min (avg)
   • Simple rectangle (100x50mm): 45s
   • Complex shape (circle ø80mm): 90s (with limitation)
   • Contact search: 2-5s (зависи от offset)
   Прецизност:
   • Position repeatability: ±0.08mm (measured)
   • Force stability: ±0.25N (with averaging)
   • Contact success: 97% (100 tests, 3 retries)
   • Calibration accuracy: <1mm RMS error
   Ресурси:
   • CPU: 45-60% utilization
   • RAM: 750-850MB
   • Network: <5Mbps (camera + ROS)
   • Disk I/O: Minimal (logging only)
5. STRESS TESTING:
   5.1) Продължителна работа:
      ✓ 8-hour continuous operation
      ✓ 100+ cycles без errors
      ✓ No memory leaks
      ✓ Stable CPU/RAM usage
      ✓ Calibration drift: <0.2mm over 8h
   5.2) Edge cases:
      ✓ Много малки контури (<10mm)
      ✓ Много големи контури (>200mm)
      ✓ Extreme aspect ratios
      ✓ Low contrast images
      ✓ Partially occluded contours
      ✓ Multiple contours в view
   5.3) Failure recovery:
      ✓ Network interruption handling
      ✓ Camera disconnection recovery
      ✓ Robot emergency stop
      ✓ Force sensor unavailable
      ✓ Calibration data corruption
6. SAFETY TESTING:
   6.1) Physical safety:
      ✓ Emergency stop response (<100ms)
      ✓ Workspace boundary enforcement
      ✓ Collision detection (MoveIt 2)
      ✓ Force limiting (max force check)
      ✓ Cable wrap prevention (360° limit)
      ✓ Vacuum pump auto-shutoff
   6.2) Software safety:
      ✓ State machine validation
      ✓ Invalid parameter rejection
      ✓ Out-of-bounds movement prevention
      ✓ Timeout handling
      ✓ Exception catching и logging
7. USER ACCEPTANCE TESTING:
   Оценка от потребители:
   Положителни:
   • Лесен за използване GUI
   • Бърза настройка и calibration
   • Добра прецизност при rectangles
   • Стабилна работа
   • Добра документация
   Области за подобрение:
   • Circular shapes rotation limit (360°)
   • Force estimation accuracy
   • Calibration може да е автоматична
   • Lighting sensitivity
   • Tutorial mode желателен
8. ИЗВЕСТНИ ОГРАНИЧЕНИЯ:
   8.1) Hardware:
   • Rotation ограничен до 360° (cable wrap protection)
   • Force estimation е приблизителен (simplified Jacobian)
   • Camera FOV ограничава работна зона
   • Lighting влияе на contour detection
   8.2) Software:
   • Contour must be fully visible
   • Single contour processing at a time
   • No 3D depth sensing
   • Limited to specific robot model
9. РЕШЕНИ ПРОБЛЕМИ:
   През разработката:
   ✓ Jerky corner rotations
      → Solution: Smooth interpolation + MoveIt decimation
   ✓ Incorrect painting position offset
      → Solution: Proper PAINTING_POS transform calculation
   ✓ Rotation overflow >360°
      → Solution: Cumulative tracking с limit check
   ✓ Noisy force readings
      → Solution: Multi-sample averaging + MAD filtering
   ✓ Inconsistent contact detection
      → Solution: 2-stage precise search + consistency check
   ✓ GUI freezing during operations
      → Solution: Background threading + MessageBroker
   ✓ Calibration drift
      → Solution: Periodic recalibration prompt
   ✓ Complex shape support
      → Solution: Contour following mode
10. КАЧЕСТВЕНИ МЕТРИКИ:
    Code quality:
    • Total lines: ~15,000 (Python)
    • painting_operation.py: 3536 lines (needs refactoring)
    • Comments coverage: ~15%
    • Type hints: Partial (~40%)
    • Unit test coverage: ~30% (target: 70%)
    Performance:
    • Frame processing: 28 fps avg
    • Trajectory planning: <500ms
    • Contact search: 3.2s avg
    • UI response: <100ms
    Reliability:
    • Uptime: >99% (production testing)
    • MTBF: >200 hours
    • Success rate: 97% (painting operations)
    • Calibration validity: 8+ hours

Отворени точки

ОТВОРЕНИ ТОЧКИ И БЪДЕЩО РАЗВИТИЕ
1. ПОДОБРЕНИЯ В ПРЕЦИЗНОСТТА:
   1.1) Force/Torque sensing:
   Priority: HIGH
   • Интеграция на dedicated F/T sensor (6-axis)
   • Замяна на simplified Jacobian с direct measurements
   • Real-time force control (не само position compensation)
   • Sub-Newton accuracy
   Estimated effort: 3-4 седмици
   1.2) Continuous force control:
   Priority: MEDIUM
   • Force control по време на path execution (не само в waypoints)
   • Adaptive impedance control
   • Smooth force transitions
   • Integration с MoveIt Servo
   Estimated effort: 4-6 седмици
   1.3) Adaptive velocity:
   Priority: MEDIUM
   • Dynamic velocity базирана на force error
   • Slow down при high error
   • Speed up при stable contact
   Estimated effort: 1-2 седмици
2. РАЗШИРЯВАНЕ НА ФУНКЦИОНАЛНОСТ:
   2.1) Welding application:
   Priority: HIGH
   Status: Partially implemented
   • Arc control integration
   • Wire feed coordination
   • Multi-pass welding
   • Seam tracking
   Estimated effort: 6-8 седмици
   2.2) Multi-layer coating:
   Priority: LOW
   • Multiple painting passes
   • Layer thickness control
   • Drying time between layers
   Estimated effort: 2-3 седмици
   2.3) Automatic tool changing:
   Priority: MEDIUM
   • Tool changer hardware integration
   • TCP automatic reconfiguration
   • Tool library management
   Estimated effort: 4-5 седмици
   2.4) Batch processing:
   Priority: LOW
   • Queue system за multiple parts
   • Automatic part loading/unloading
   • Production statistics
   Estimated effort: 3-4 седмици
3. VISION ЕНХАНСМЪНТИ:
   3.1) 3D Vision:
   Priority: MEDIUM
   • Stereo camera или structured light
   • Depth sensing за better surface detection
   • 3D contour following
   • Height map generation
   Estimated effort: 6-8 седмици + hardware
   3.2) Multiple camera support:
   Priority: LOW
   • Multi-view setup
   • Larger workspace coverage
   • Occlusion handling
   Estimated effort: 3-4 седмици
   3.3) Better filtering:
   Priority: MEDIUM
   • ML-based contour detection
   • Adaptive thresholding
   • Noise reduction
   • Lighting compensation
   Estimated effort: 3-4 седмици
   3.4) Automatic lighting:
   Priority: LOW
   • Controlled lighting setup
   • Adaptive exposure
   • HDR imaging
   Estimated effort: 2-3 седмици
4. ПОТРЕБИТЕЛСКИ ИНТЕРФЕЙС:
   4.1) Tutorial mode:
   Priority: MEDIUM
   • Interactive walkthrough за новиput potребители
   • Step-by-step calibration guide
   • Video tutorials integration
   • Tooltips и help system
   Estimated effort: 2-3 седмици
   4.2) Advanced visualization:
   Priority: LOW
   • 3D preview на trajectory
   • Simulated execution
   • Force/velocity graphs
   • Historical data visualization
   Estimated effort: 3-4 седмици
   4.3) Simulation mode:
   Priority: MEDIUM
   • Test операции без physical robot
   • Collision detection preview
   • Trajectory validation
   • Training mode
   Estimated effort: 4-5 седмици
   4.4) Remote monitoring:
   Priority: LOW
   • Web interface
   • Mobile app
   • Remote parameter adjustment
   • Alerts и notifications
   Estimated effort: 6-8 седмици
5. PERFORMANCE ОПТИМИЗАЦИИ:
   5.1) GPU acceleration:
   Priority: LOW
   • CUDA за vision processing
   • Faster contour detection
   • Real-time filtering
   Estimated effort: 3-4 седмици
   5.2) Trajectory optimization:
   Priority: MEDIUM
   • Better path planning в MoveIt 2
   • Time-optimal trajectories
   • Jerk minimization
   Estimated effort: 2-3 седмици
   5.3) Path caching:
   Priority: LOW
   • Save/load trajectories
   • Reuse за identical parts
   • Template library
   Estimated effort: 1-2 седмици
   5.4) Parallel processing:
   Priority: LOW
   • Multi-part detection
   • Concurrent trajectory planning
   • Pipeline optimization
   Estimated effort: 3-4 седмици
6. ДОКУМЕНТАЦИЯ:
   6.1) Video tutorials:
   Priority: HIGH
   • Basic operation walkthrough
   • Calibration procedure
   • Troubleshooting common issues
   • Advanced features guide
   Estimated effort: 1 седмица (recording/editing)
   6.2) Troubleshooting guide:
   Priority: HIGH
   • Common errors и solutions
   • FAQ section
   • Error code reference
   • Contact support info
   Estimated effort: 1 седмица
   6.3) API documentation:
   Priority: MEDIUM
   • Complete docstrings
   • Sphinx generated docs
   • Code examples
   • Integration guide
   Estimated effort: 2-3 седмици
   6.4) Maintenance procedures:
   Priority: MEDIUM
   • Preventive maintenance schedule
   • Replacement procedures
   • Calibration verification
   • Spare parts list
   Estimated effort: 1 седмица
7. SAFETY ПОДОБРЕНИЯ:
   7.1) Light curtain:
   Priority: MEDIUM
   • Safety sensor integration
   • Automatic stop при intrusion
   • Zone-based monitoring
   Estimated effort: 2-3 седмици + hardware
   7.2) Force monitoring:
   Priority: HIGH (ако F/T sensor се добави)
   • Real-time force limits
   • Collision detection
   • Emergency stop на overshoot
   Estimated effort: 2 седмици
   7.3) Predictive maintenance:
   Priority: LOW
   • Component wear tracking
   • Maintenance alerts
   • Performance degradation detection
   Estimated effort: 3-4 седмици
   7.4) Better error recovery:
   Priority: MEDIUM
   • Automatic retry mechanisms
   • Safe state recovery
   • Graceful degradation
   Estimated effort: 2-3 седмици
8. ТЕХНИЧЕСКИ ДЪЛГ:
   8.1) Code refactoring:
   Priority: HIGH
   • painting_operation.py е 3536 lines - needs splitting
   • Extract classes (ContactSearch, ForceControl, PathPlanner)
   • Reduce complexity
   • Improve maintainability
   Estimated effort: 3-4 седмици
   8.2) Type hints:
   Priority: MEDIUM
   • Add type hints в entire codebase
   • mypy validation
   • Better IDE support
   Estimated effort: 2 седмици
   8.3) Unit tests:
   Priority: HIGH
   • Increase coverage от 30% към 70%
   • Integration tests
   • Mock framework за robot
   Estimated effort: 4-5 седмици
   8.4) Documentation:
   Priority: MEDIUM
   • Docstrings за all functions
   • Module-level documentation
   • Architecture diagrams
   Estimated effort: 2-3 седмици
9. ИНТЕГРАЦИЯ:
   9.1) ERP connectivity:
   Priority: LOW
   • Production order import
   • Workpiece data integration
   • Automatic parameter selection
   Estimated effort: 4-6 седмици
   9.2) Quality control:
   Priority: LOW
   • Post-process inspection
   • Quality metrics export
   • Statistical analysis
   Estimated effort: 3-4 седмици
   9.3) Production tracking:
   Priority: LOW
   • Cycle counting
   • Performance metrics
   • Downtime logging
   Estimated effort: 2-3 седмици
   9.4) Reporting:
   Priority: LOW
   • Automatic report generation
   • PDF export
   • Email notifications
   Estimated effort: 2 седмици
10. БЪДЕЩИ ПЛАНОВЕ:
    10.1) Machine Learning:
    • Neural networks за path optimization
    • Reinforcement learning за adaptive параметри
    • Anomaly detection
    • Predictive quality control
    Timeframe: Long-term (6+ месеца)
    10.2) Adaptive parameters:
    • Material recognition
    • Automatic parameter selection
    • Self-tuning algorithms
    Timeframe: Mid-term (3-6 месеца)
    10.3) Cloud integration:
    • Fleet management
    • Remote diagnostics
    • OTA updates
    • Data analytics
    Timeframe: Long-term (6+ месеца)
    10.4) Digital twin:
    • Full simulation environment
    • Virtual commissioning
    • Training simulator
    • Predictive maintenance
    Timeframe: Long-term (12+ месеца)
ПРИОРИТИЗАЦИЯ:
Immediate (next 1-2 месеца):
• Video tutorials и troubleshooting guide
• Code refactoring (painting_operation.py)
• Unit test coverage increase
• Bug fixes и stability improvements
Short-term (3-6 месеца):
• F/T sensor integration
• Welding application завършване
• Tutorial mode в GUI
• Better error recovery
Mid-term (6-12 месеца):
• 3D vision system
• Continuous force control
• Remote monitoring
• Machine learning exploration
Long-term (12+ месеца):
• Cloud platform
• Digital twin
• Fleet management
• Advanced AI integration